Model WC4WA1313H specification

Pump: model WIP4

High pressure pump type: intensifier.

Maximum  pressure: 280Mpa (40kpsi).

Maximum water flow rate 3.0L/minute (0.8Gal/min)

Pump cooling method: forced water flow, electrical chiller is optional.

Digital display of working pressure.

Water Filter: 2-stage water filtration included.

Minimum supply water pressure to pump: 70psi (5kg/cm2)

Internal water pressure booster pump.

Approximate size: 60" x 32" x 55" (1600x800x1400mm)

Power source: 3 phase, 220VAC 60Hz or (380VAC, 50Hz),

22kw, 30 HP. System starting current is about 100Amp.

Cutting table:

Style: overpass

water tank made of stainless steel.

Effective cutting area: 4ft x 4 ft (1300x1300mm)

Maximum Z-axis travel: 6" (150mm)

X-Y axis motion system: ball screw, linear motion guide

X-Y axis motion drive: AC servo motor

Z axis motion: AC motor with speed reducer.

X-Y axis position accuracy: +/- 0.0008"(0.02mm).

X-Y axis position repeatability: +/- 0.0004"(0.01mm).

Best cutting tolerance: +/- 0.006" (0.15mm).

Maximum X-Y axis travel speed: 32ft/min (10m/min).

Cutting head:

Pneumatically operated on/off valve, self-aligned cutting head. Maximum Abrasive flow rate:  0.5kg/min (1lb/min). Compressed air source is necessary. An air compressor is optional item from manufacturer. As an option, the cutting head can manually tilt in both x and y directions.

This machine can cut all kinds of metal, marble, granite, glass, ceramic tile, rubber, plastic, virtually any materials. This machine can cut metal up to 40mm(1.5”) thick.

CNC controller:

Advanced industrial grade computer. The controller is capable of X-Y axis interpolation movement. Accelerate or decelerate the motion and high speed travel from point to point. Simulation without motion or dry run with axis motion but no jetting of water/abrasive are features included. Error stop and trace repeat are also very useful features. Out of range warning and hard stop to prevent damage of the cutting head. Emergency shut off switch is provided. Remote control is provided to move X-Y-Z axis when operator is away from the controller tower.

Controller Tower					Remote Control

DXF file compatible:

The controller is compatible to dxf file format. Users can save design drawing in .dxf from AutoCAD and read the file into the controller. Controller will post the dxf file to G-code.  The controller can also read G-code generated from other CAM software. Controller software interface is based on Microsoft Windows 98/XP/VISTA.

Advanced CAD/CAM software (optional):  for design drawing, define machine path and lead in/out, nesting, tool offset, and generate G code. Features are similar to AutoCAD, and with rich features of CAM.

Automatic abrasive feeder (optional):

This equipment is highly recommended.

- Requires 70psi pneumatic source

- Capacity 40 Gallon.

- Self detecting empty/full status of reservoir and start/stop feeding.

- Model number ADS-04-170.

Accessories included in standard package:

High pressure tubing

Water hose

Electrical cables

1 pcs nozzle, 2 pcs orifice, 1 tube of blue goop.

1 set of special tools for pump maintenance.