The model WC6WA6030H abrasive cutting system:

Pump:

High pressure pump type: dual reciprocal intensifier.

Maximum pressure: 400Mpa (60kpsi).

Maximum water flow rate 7L/minute (1.6Gal/min)

Pump cooling method: water.  Oil chiller is optional.

Water Filter: 2-stage water filtration included.

Approximate size: 60" x 32" x 55" (1600x1200x1400mm)

Power source: 3 phase, 380VAC, 50Hz, (or 220V 60Hz) 75kw, 100 HP. Motor starting current requirement: 250Amp

CNC Cutting table:

Style: gantry.

Effective cutting area: 3000x6000mm.

Maximum Z-axis travel: 6" (150mm).

X-Y axis motion system: rack-pinion/ball screw, and

linear motion guide.

X-Y axis motion drive: AC servo motor.

Water tank made of stainless steel.

Z axis motion: AC motor with speed reducer. (servo motor optional)

Cutting tolerance: 0.3mm/m.

Repeat accuracy +/-0.1mm/m.

Maximum X-Y axis travel speed 10m/min.

Maximum work piece weight: 6000kg.

Cutting head:

Cutting head has pneumatically operated on/off valve. Compressed air source is necessary. An air compressor is optional item from manufacturer. Capable of cutting up to 50mm mild steel and 25mm stainless steel. Optional feature: bevel cutting up to 45 degree with straight lines parallel to X or Y. Dual cutting head is also optional.

CNC controller:

Advanced industrial grade computer. The controller is capable of X-Y axis interpolation movement. Accelerate or decelerate the motion and high speed travel from point to point. Simulation without motion or dry-run with axis movement but no jetting of water/abrasive are features included. Error stop and trace repeat are also very useful features. Other features include out of range warning and hard stop to prevent damage of the cutting head.  Emergency shut off switch is provided.  CNC system is based on Windows operating system winXp/Vista and is very easy to learn and operate. Controller software interface runs on Microsoft Windows 98/XP/VISTA. Both metric and imperial versions are available.

DXF file compatible:

The controller is compatible to dxf file format. Users can save design drawing as .dxf from AutoCAD and read the file into the controller. Controller will post the dxf file to G-code.  The controller can also read G-code generated from other CAM software.

User Interface of Controller software

CAD/CAM software (optional):

Automatic abrasive feeder:

- Requires 70psi pneumatic source

- Capacity 40 Gallon.

- Self detecting empty/full status of reservoir and start/stop feeding.

- Model number ADS-04-170.

Accessories included in standard package:

High pressure tubing, Water hoses, Electrical cable, 1 pcs nozzle, 2 pcs orifice, 1 tube of Blue Goop.

1 set of special tools for pump maintenance.

Spare parts kit (optional).

Including 10 nozzles, 20 orifices, 1 HP seal kit, 1 LP seal kit, 1 HP cylinder, 1 check valve body, 1 check valve repair kit, 1 on/off valve repair kit.